# 2017 Late Model Rules

2017 changes in red

A. Eligible cars and bodies

1. All competing cars will be full sized ,stock American manufactured passenger car bodies that conform to the current ABC body rules are allowed. 2004 or newer

Current ABC body rules apply unless otherwise specified herein. Refer to ABC rulebook body guidelines will be posted at    www.abcbodies.com

Spoilers- 6-1/2” x 60” required. Clear spoiler only, must have slot in center for template

Wheelbase 103 plus or minus 2” Front and rear tread width is a maximum of 66 inches. NO 25 lb deduction for 65 inch or under. Over 66 inch's Not Allowed.

2. No attempt to get any aero advantage allowed, No panning of bottom of car except from radiator to front bumper maximum width 29 inches. and between frame rails. For air intake to radiator. ABC rules MEASUREMENT “A” must be a minimum of 11.5 inches and nose measurement must be 20 inches minimum from hood to bottom of nose.

3. Five Star Bodies or flat 12 inch side vent windows only

4, 3 window braces front and 2 rear window braces. Must be approved.

5. No holes in any body panels or windows to exhaust air. All holes or vents must remain open for qualifying except nose panel.

6. The ABC Referee will be the official method of body measurement of the Grundy County Speedway including TREAD WIDTH.

##### B. Engines

1. Block must be cast iron.

2. No 18 degree or SB-2 Chevrolet heads.

3. Minimum crank height is 10 inches measured from the center of the forward crank bolt or back of crankshaft.

4. All GM cast iron engines must be located so that the centerline of the forward most spark plug hole is no more than 2 inches back from the center of the upper ball joint. Ford, Mopar, and LS engines may be located so the center of the forward most spark plug hole of the engine is a maximum of 4 inches rearward of the centerline of the upper ball joint. Cars with offset upper control arms the measurement will be from centerline of upper control arm to centerline of the forward most spark plug hole.

5. Antifreeze is strictly prohibited

C. ACE Type Engines

Ace cylinder heads and intake manifolds must be unmodified. machining,cutting,grinding,abrasive blasting,use of chemicals, or any alterations to change or alter the cylinder head or intake manifold from its as cast state is prohibited

Must be able to sell heads, complete for $2500.00 (hardware, valves, valves springs, retainers, keepers and guide plates.) Heads must be stock out of box.

Valves 11/32 valve stem or 5/16 valve stem may be used.

No titanium valves allowed.

All valve spring sizes must be 1.55 max. ISKY 9935 valve spring 1.56 O.D. is legal

No shaft rocker arms allowed except on Mopar engines.

Steel or titanium valve spring retainers are permissible.

Maximum 4 stage oil pump.

May have one extra water line per head.

Valve job may be blended into combustion chamber 3/8 inch from seat.

1. Ace Engine Manifolds

Any production type intake manifold allowed - provided it is readily available to all competitors from local race part suppliers. (maximum cost $375.00) maximum height of manifold is 7.25" (including any carb spacer and gaskets) the manifold height will be measured from the base of carb to top of cylinder block. Only one flat gasket with a maximum of .120 may be used between intake manifold and cylinder head - no spacer or wedge type gaskets allowed. No additional material may be added to manifold. No grinding or polishing of any part of the manifold - except you may match port the runners a maximum of 1".

2. Ace Engine Pistons

Flat top pistons only - no part of piston may protrude above top of cylinder. (maximum) compression ratio 10.5 to 1 (10.510 is illegal). Maximum engine displacement for GM and Ford is 358 c.i. inches, Dodge will be 360 c.i. and minimum 350 c.i. for GM, 346 c.i. for Ford.

3. Ace Engine Camshaft

The max lift on any roller cam is .625. Duration rule is 270 at 50 thousandths. No mushroom type lifters. Inlayed cams are prohibited. The maximum rocker ratio is 1.6 to 1. Rev kits of any type are prohibited. Only steel push rods (titanium, aluminum or graphite are prohibited). No roller bearing camshaft journals. Magnetic steel lifters no ceramic.

4. Ace Engine Connecting Rods

Only  approved steel rods allowed. No titanium, aluminum, graphite or stainless steel. Rods using 3/8" bolts are allowed.

5. Ace Engine Blocks

Must be standard factory production cast iron. (only 010 or bowtie approved). No aluminum blocks permitted. No altering of engine block permitted.. Absolutely no grinding or lighting of blocks . The use of aftermarket blocks will be allowed in ace engines.  No big bore short stroke ace engines will be allowed. No carbon composite or light weight blocks allowed.

6. Ace Engine Crankshaft

Standard steel type only, minimum allowed weight of 43 lbs. (or stock type for block used)stock angle crankshaft allowed. Lightweight, undercut counter weight crankshaft are prohibited. No Honda journal crankshafts. Stroke 3.400 min to 3.500 maximum.

Minimum 1.980-rod journals or any under sized journals under factory dimensions.

7. Ace Inspection

A 1.5” plug must be installed in the oil pan for inspection purposes. This hole must be directly under or side of the rod journal. If a windage tray is used, a hole must be provided in line with the hole in the oil pan. Cylinder head removal after any race may be required for inspection purposes.

D. 9 to 1 Aluminum Head Engines

1. Engine Block

Must be cast iron. No carbon composite or light weight blocks allowed. Must be stock appearing.

2. Crankshaft

Standard steel type only, minimum allowed weight of 38 lbs., stock angle crankshaft allowed.

3. Pistons

Flat top pistons - no part of piston may protrude above top of cylinder.

9 to 1 aluminum headed motors will have a 9.5 to 1 compression ratio (a ratio of 9.51 to 1 or higher will not be allowed). Maximum engine displacement of 358 c.i. and minimum 347 c.i. Aluminum headed motors may use dished or inverted dome pistons.

4. Connecting Rods

Only approved steel rods allowed. No titanium, aluminum, graphite rods or stainless steel are allowed.

5. Camshaft

Only steel push rods (titanium, aluminum or graphite are prohibited). 9 to 1 aluminum headed engines are allowed roller cams and rev kits.

6. Heads

All cast in part numbers must remain unaltered. Painting and /or coating of the heads will not be permitted.

No 18-degree GM heads.

For all 9.5 compression motors the cylinder heads must be acceptable to  officials and meet the following requirements: Only steel or titanium valves will be permitted. Only magnetic steel valve springs will be permitted and only 2 valves per cylinder will be permitted there are no valve size restriction internal polishing and porting will be permitted spark plug holes must remain in stock location valve angle must remain within 2 degrees of stock angle valves must remain in the stock location in relation to the cylinder bore centerline.

7 Intake Manifolds

No fabricated intakes must be made of aluminum. Only one flat gasket with maximum of .120 may be used between intake manifold and cylinder head no spacer or wedge type gaskets allowed.. May be polished and ported.

8.  Added directional devices will not be permitted inside the intake manifold. Air holes will not be permitted to be opened in the intake manifold. Painting and /or coating of the intake manifold will not be permitted.

9. No engine part maybe composite. All part numbers must remain on all engine parts No crank fire ignitions

E. Iron Headed Concept Engine

1. heads: Cast Iron heads GM part # 1401158, 10134392 (casting #10411034)

Ford M-6049-N351 or M6049-E351 only. The heads must remain as manufactured. Valves, rocker studs, head bolts and spark plugs may not be relocated. Porting, Port matching and or polishing is not permitted. Intake port runner volume must be 187cc or less. Combustion chamber may be polished but must retain stock configuration. No angle milling, valve angle must remain 23 degrees. Heads can be flat milled a maximum of .050 inch Combustion chamber 62cc minimum volume. 10.8 maximum compression ratio. A three angle valve job with bowl cutting and minor blending is permitted. No work permitted to ports above bottom of valve guide.Valve guide bottom may not be shortened or narrowed. Any evidence of chemical treating, acid dipping, acid flowing, abrasive blasting or other alterations to the original as cast port or the addition of material to the ports or combustion chamber will cause those cylinder heads to be declared illegal! Ports must pass plug check

2.  Valves: Maximum valve size 2.02 intake, 1.60 exhaust. No titanium valves

3.  Springs: Maximum valve spring diameter 1.56 inches

4.  Rockers: Stud mounted rockers with 1.60 maximum ratio no shaft mounted rockers allowed

5.  Blocks: Block must be GM, Ford, and Dodge production 350, 4.040 maximum bore. No altering of engine block permitted. No angle cutting of block. No aluminum blocks are permitted. Minimum displacement 350.00 cubic inches, maximum displacement 358.00 inches.

6.  Crankshaft: 3.50 stroke maximum. No 180 degree cranks, No titanium

7.  Connecting rods: Only steel rods allowed(titanium, aluminum, graphite, or stainless rods are prohibited)

8. Pistons: flat top only. No part of piston may protrude above top of cylinder.

9. Camshaft: Only flat tappet cam with stock diameter lifters allowed; (roller, mushroom type lifters and inlaid cams are prohibited) Rev kits of any type are prohibited; only steel push rods allowed (titanium, aluminum,graphite prohibited)

10. Intake: Any aluminum intake readily available to all competitors that maintains a maximum of 7” of manifold height. The manifold height will be measured from the base of the carburetor to the top of the cylinder block. Only one flat gasket with a maximum of .120 may be used between intake manifold and cylinder head. You may port match 1” into manifold. No other porting, grinding, sanding, blending,chemical porting, abrasive blasting or extrude honing. Addition of any material or any alteration from original is forbidden.

F. Carburetors and Spacer Plates

All carbs must pass all Grundy County Speedway gauges and specs.

Boosters must be stock appearing and as cast for carbs style and no extra holes may be drilled. May not be tapered. Must also be in stock location in body. No modifications of boosters allowed. Spacers can be 1 ½” max and bores must be perpendicular to the base. No tapered spacers.

These parts must be Grundy County Speedway  gauge legal.

Throttle bores ,Boosters and booster legs ,Throttle plates ,Throttle shafts &

Main body

The Ultra Series Holley Carb is not allowed.

Metering blocks must be stock as cast for carb style and no extra holes may be drilled. Block may be plugged and may be machined but must remain stock appearing no aftermarket blocks.

G. Fuel and Fuel Cell

No oxygen bearing or performance enhancing additives may be introduced into the inductions or fuel supply, either at the fuel cell or upstream in the system. Violations will result in immediate disqualification from the event; forfeiture of owner and driver points, and monies/contingencies earned for the event Fuel cells with rubber bladders fuel cell plates or fuel cell tubs are mandatory. Fuel cell protector plate 1/8 thick steel must be mounted on outside of frame rails. The plates must cover the sides and rear of the fuel cell and be official approved. Fuel cell minimum height 8 inches. Fuel cell must be banded both ways with two steel straps each way. 1-inch minimum straps. Fuel cell tub 1/8 thick steel with one-inch lip. Front, bottom and rear will be one piece. The top of the box will use current 18 or 20 gauge top with 1 inch by 1/8 steel straps with two in each direction. All fuel cell cans must be magnetic steel. All fuel cells must have check balls in place. Racing pump fuel only any over the axle style rear tail style chassis must use approved 1/8 inch magnetic steel fuel cell can. Any chassis with incorrect fuel cell can will be asked to change or be disqualified. The cell must be bolted in with a minimum of 14-3/8 bolts with flat washers on top and lock washers on bottom. The top for this cell will be 18 gauge steel with steel straps in both directions. A sonic tester will be used to check fuel cell can thickness.

The fuel cell must be mounted behind rear end. The use of an Oberg Fuel Shut Off valve mounted in the fuel line coming out of the fuel cell within the first 12 inches from cell is mandatory.

H. Weight Combinations-

This year we will be testing a new weight, left side percentage combination for cars using a conventional setup with no bump stops and rebound adjustable shocks only that we will be using in the 2018 season

All cars will be 58% max left side weight.

2725 lbs- Ace engines with 4412 2bbl Holley- 500 cfm

2725 lbs- Mcgunegill equalizer sealed with 4412 2bbl Holley- 500 cfm. 7600 chip

2725 lbs- Ford sealed S374D with 4412 2 bbl Holley-500cfm 7600 chip

2725 lbs- Wegner sealed engine with 4412 2 bbl Holley- 500 cfm

2750 lbs- 9 to 1 aluminum engines with 4412 2 bbl Holley- 500 cfm

2725 lbs- Hamner sealed engine with 4412 2 bbl Holley- 500cfm. 7600 chip

2775 lbs  Concept  with 750 cfm (no HP) 4 bbl

2675 lbs Crate engines with 650 cfm 4 bbl holley. 6300 chip ford, 6400 chip chevy

2650 lbs Crate GM 604* with 650 cfm 4 bbl holley. 6400 chip

* GM part # 88959604 with original seals, No updates or has not been rebuilt

## All weight must be solid LEAD ,No tungsten, all lead weight must be securely fastened, no lead weight allowed in drivers compartment. Lead must be painted white with car # painted on each piece

##### ADJUSTMENTS

Allow 1 lb per race lap

Add 25 lbs for every body infraction including low roof and nose heights

## Add 25 lbs for non approved fuel cells and plates

PROTEST remove crate engine, run on dyno and return $1000.00 Refund to

Owner or protester minus expenses

### Tread width on all late models 66 inch maximum.

I. Mufflers and Headers

Mufflers are required for competition.  95 decibels.  All exhaust highly recommended to exit under car to meet this requirement. All exhaust systems must have mufflers that are not tampered with or hollowed. No custom high dollar headers (no lightweight stainless, titanium or inconeal) allowed. Any collector may be used with out a cone style inserts. No one off custom header allowed.

J. Air Intake

1. Forward intakes are not allowed. Air boxes are permitted. The back of the air box must be flat or must be stock Five Star part.

2. No devices for directing the flow of the air into the air cleaner or air box are permitted.

3. No additives allowed in air filter.

4. You may not grab or funnel air into air box.

K. Clutch

1. The 5.5 inch or bigger will be the only clutch allowed. Max price MSRP. $1600

2. Absolutely no carbon fiber or poly clutches allowed.

3. Bell housing must have a minimum 2 1/2” hole at bottom (to allow a clear view of clutch). No slipper or centrifugal clutches allowed.

L. Transmissions

1. Bert or Brinn style transmissions are allowed.

2. No bottom load transmissions.

3. Must have two forward and 1 reverse working gears minimum.

4. One single lever shifter. No push and pull rods.

5. Must be self starting.

M. Brakes

1. All cars must have functioning brakes on each wheel.

2. Maximum 4 piston brake calipers.

3. Fixed mounted or floating rotors only. Steel rotors only. Maximum $500 limit on brake calipers for all cars.

4. Must also be sold on open market.

5. No other material may be used other than steel for rotors. No Carbon Fiber or any other material.

N. Shocks

1. Maximum cost on racing shocks is MSRP $850 for a complete unit.  All shocks must be Grundy County Speedway approved

2. One shock and coil spring per wheel and or corner.　 Use of eliminators is allowed.

3. No shock blankets or covers allowed

4. No electronic shocks permitted.

5. The use of bump springs will be allowed. A bump spring must act like a bump rubber and may not be larger than 2 inches in diameter and 3.75 inches tall. No other types of bump springs may be used. The car may have 4 springs, one for each wheel and 4 shocks, one for each wheel. A bump spring should look like the ones sold at www.bumpspring.com as of 12/17/13. A bump spring maybe used n a remote shock eliminator type set up, but again must look and act like a o bump rubber.

6. All springs for suspension must be magnetic steel including bump springs.

7. Shocks must be mechanical and no part of the shock or suspension may utilize electricity. No Magnetic Shocks, No air blow up or  non conventional style bump stops.

O. Suspension

Coil over or leaf allowed. No computer or hand operated controlled suspension. No aluminum axle tubes, No titanium axle shafts, No aluminum rotors, No carbon fiber rotors.

P. Roll Cage Construction

Following is the minimum specification requirements for roll cage construction  Grundy County Speedway officials reserve the right to sonic test any or all, structural chassis members at any time  Structural chassis member(s) found in violation of minimum requirements render that chassis ineligible for competition until minimum standards are met or exceeded. Drilling holes to lighten any part of the body, chassis, suspension or bolts is not permitted. Only steel round; rectangular or square tube is approved for roll cage or chassis construction of any main or supporting sub-structures. Wall thickness; size and/or diameters are specified where necessary. A four-point (4) roll cage structure utilizing a minimum 1.75-inch x .090-inch (1-3/4”x.090”) o.d. d.o.m. steel tubing is mandatory. The entire structure must be welded to the primary frame structure with a minimum of four (4) horizontal driver side door bars. A minimum of 2” x 3” x .095” wall steel tubing is mandated for main frame rails. Main frame rails are identified as mid-section rails. Main frame rails and side rails must be located within the normal tread width of the car. A minimum of 2” x 3” x .083” wall steel tubing is optional for front clip rails, rear clip or kick-up rails. No material substitution permitted. Roll cage structure must be braced to the front frame stub, with the hoop section surrounding the engine compartment; running rearward with diagonal member’s connection to the rear frame section. Nose, right side kick outs and rear bumper cover supporting structures must be a minimum 1.500-inch x .063- inch o.d. steel tube. No material substitution permitted. Absolutely no aluminum allowed on the structure of the chassis.

Q. Driver Side Door Plates

Left side driver support bars or plates are mandatory. See option a or b listed. No material substitution is permitted. All support bars or plate installation is subject to approval. All door bars need to be plated. All plates must be steel.

See illustration a.1

Plan A – 0.125-inch, 1/8” solid steel plate bolted to the left side door portion of the roll cage.

Doorplate must be bolted to the roll cage using a minimum of six (6) each 3/8” (.375-inch) aircraft quality bolts and washers. Welding of the plate to the roll cage is prohibited.

Plan B – minimum 0.125-inch (1/8”) thickness steel plate must be welded to the space between each left-side door bar.

Offset chassis right side door bars commonly called the outrigger or the kick-up bar, must be

constructed of a minimum 1.250-inch x .065-inch wall round or square steel stock. All supporting substructure must be constructed of 1-inch x .063-inch wall round or square steel stock. No material substitutions permitted.

R. Driveshaft

The driveshaft shall be made of steel or aluminum. Carbon-fiber driveshafts are not permitted.

Containment hoops (2 required), constructed of a minimum 0.1875-inch thick steel, are mandatory and

The forward hoop Must be 4-5 inches minimum behind front yoke.

S. Front Suspension

Independent front suspension with articulated upper and lower control arm(s) is mandatory. The type of shock absorbers and suspension springs are optional. One (1) shock absorber and spring per corner of the car is permitted. Front suspension adjustment must be done from under the car or by lifting the hood. No holes in the hood, fenders or other body parts from the windshield forward to adjust front suspension component(s) are permitted. No suspension adjustment devices are permitted in the driver’s compartment area Or in reach of driver at any time in car. Knob-type brake bias adjusters are recommended. Weight transfer or suspension adjustment devices, adjustable while the car is under way are prohibited. Spring rubbers are permitted and must be removed manually. No removal devices may extend out side the body of the car or be accessible to the driver in the drivers compartment. Manual or power steering maybe used. No electronic power steering. Shocks must be mechanical and no part of the shock or suspension may utilize electricity.

T. Rear Suspension

Non-independent, live axle type rear suspension is mandatory. Rear ends may be quick-change, minimum 8 inch ring gear. With full-floating hubs or 9-inch Ford type. Rear axle tubes must be steel. No open tube rear ends permitted. Maximum rear camber is + or - 1 degree measured with the rear axle level. Material used for rear end center section is at the discretion of the team, but hub pins must be steel.  Remote rear suspension adjusters are permitted when accessible through the rear window. A maximum of three (3) one-inch (1”) diameter holes are permitted in the rear window. Each hole can allow access to one adjustment device only. No adjuster may extend forward of the rear window area. All pumps used to circulate fluid for the purpose of cooling the rear end, must be mounted in the center of the car.No bird cage setups of any kind.No part of trailing arm mounting bracket may rotate or move

U. Wheels

Approved wheels must be 15-inch diameter; five-lug (5) steel; 5” x 5” hub or wide 5 patterns; 10-inch rim width. Bleeder and/or pop-off valve devices are not permitted; alteration or defacing of wheel identification numbers; labels; code numbers or serial numbers is not permitted. Wheel(s) failing this criteria will be ineligible for competition.

V. Wheel Studs and Spacers

A minimum of five (5) lug nuts per wheel, minimum 0.625-inch (5/8”) solid steel nuts, showing a minimum of two (2) threads through the nut, must extend through the lug nut when clamping the wheel to the hub. Wheel spacers, if used, must be made of steel or aluminum and a minimum 6.75 inches in diameter. Shims are not permitted when mounting wheel studs to hubs.

W. Safety

In all matters pertaining to safety, car owners, drivers and crewmembers must review and educate

themselves in all safety standards. It is the responsibility of the car owners, drivers and crewmembers to install, wear and maintain all safety equipment as specified by manufacturer’s instructions. This includes, but is not limited to, helmets, fires suits, racing suits, gloves, shoes, flame-resistant underwear, flame-resistant head sock, head and neck restraint systems, driver’s racing seat and safety belts. Any safety infraction will deem the car ineligible for competition until the infraction has been repaired or corrected and the car re-inspected. Drivers wearing dental plates or dentures are required to remove them for any hot-track activity. Steering wheel quick release hub mandatory

All cars must have tow hooks installed (2 in front on bay bars and 2 in back on fuelcell protector bars. Must be able to support weight of car under tow.

#### X. Driver seat

All driver seats must be manufactured by a recognized manufacturer of seat and safety equipment, multi-layer aluminum seat and approved by Grundy County Speedway officials. Seats must remain “as purchased and produced”, no holes or other modifications made for weight reduction. Homemade seats or sprint car type seats are not permitted. Seat construction must be solid aluminum sheet material from the seat bottom to above the driver shoulder area; must be fully padded, with padded pelvis, rib and shoulder supports on both the left and right side. Exception – Lajoie seat where construction is such that rib supports are not required. A head restraint system, manufactured by a recognized manufacturer of seat and safety equipment, is mandatory. Bolt on systems are approved for competition. Seats are recommended to have left and right leg extensions, fully padded, running from the edge of the seat to the entrance of the foot box area.

Recommendation – a minimum 1/8” (.125-inch) thick steel plate be mounted on the front of backside of the rear hoop of the mid-section in front of the left rear wheel. Plate should extend from the horizontal shoulder bar downward the height and width of the driver seat.

A.1 seat belt and shoulder harness installation

All seat belt and shoulder harness systems must me SFI specification 16.1, type 1. Y-type shoulder belts are not approved for use. Seat belts and shoulder harness systems must have a production date within three years of the event date. A minimum five-point harness system is mandatory. Competitors using the HANS device may use a standard three-inch (3”) or the schroth racing two inch (2”) wide should strap. The Schroth Racing should strap system has been specifically designed for use with the HANS device. Schroth part numbers are profi iii-6fh; hybrid iii-h; profi iii-6h. Shoulder harness belts shall not be mounted lower than the shoulder line of the driver or 10 degrees. All lap belt and shoulder harness mounting must be done with aircraft-quality bolts and washers. See illustration a.2 and a.3

B.1 Driver Helmet

All driver helmets must reflect a Snell SA2010 certification minimum. SFI or Snell approval sticker must be visible for Grundy County Speedway officials inspection. Eye protection is mandatory at all times.

##### C.1 Left Side Window Net

Left side driver window net is mandatory. Construction must be web-type safety net with mechanical release. Net bar must be a minimum of .1875-inch (3/16”) flat steel or .375-inch (3/8”) round stock and run the entire length of the window net between mounting points. Mechanical release must be welded to the front or “a” pillar end of the bar. Spring-loaded releases are not approved for competition. Driver net must be secured in place and centered in the door area and must be secured to the upper roll cage horizontal member. Window nets must drop down. Must latch on top. No Fish net style window nets.

D.1 Fire Suppression System

A minimum five-pound (5) on-board fire suppression system, with multiple discharges point is highly recommended . Cold Fire recommended for cockpit usage. Must have gauge in view. Must be fully charged.

E.1 Driver Head/neck Restraint System and Driver Uniform

Use of head and neck restraint devices is highly recommended for all hot-track activity. Approved devices are the HANS device, LFT Technologies R3, Simpson and the Hutchens ii device. Driver uniform must be a multi-layer, full-coverage, one-piece fire-retardant uniform specifically designed for racing, fire retardant gloves, socks, underwear, and shoes.

F.1 Tires

Hoosier tires are the official tire of the Grundy County Speedway The Hoosier F40 left side and the Hoosier F 50 right side tires are the only tires allowed.

The tires you qualify on, you must run in the feature.

(4) tires first 2 weeks of racing (not your 1st  2 weeks) then 2 tires per night

tires must be purchased at the track , tires will be scanned, only scanned tires purchased at the track will be allowed in competition

Alteration of a tire(s) is not permitted and defined as changing the physical and/or chemical composition of the tire by cutting; grinding; buffing; warming; cooling or the use of chemicals whereby the tread area or the interior surfaces of the tire is changed from the manufacturer’s specifications; alteration or defacing of tire identification numbers; labels; code numbers or serial numbers. Any violation of this nature causes the tire(s) to be deemed ineligible for competition.

G.1 Traction Control and On-Board Computer or Analysis Equipment.

No equipment of this nature is permitted on any car or located in the pit area of any event and will subject the  to confiscation of equipment.

1. Only one camera pointing out front windshield allowed.　Multiple cameras not allowed.　 No computer or video analysis equipment of any kind allowed.

H.1 Ignition

All ignition systems must be 12 volts. Only one 12 volt battery maybe used at any time. Ignition boxes may be switched by Grundy County Speedway from car to car or swapped with tracks house ignition boxes. Battery kill switch is mandatory and must be located with easy access

Ignition boxes must be mounted as far right as possible, driver can not be able to touch ignition box when seated in car, RPM chip must be facing right side of car. THIS RULE WILL BE STRICTLY ENFORCED

## IGNITION BOX

THE MANDATORY IGNITION WILL BE THE CRANE CAMS IGNITION FOR ALL CARS, HI-6RC (p/n 6000-6700) PS92N coil (p/n 730-0192) Ignition Tray (p/n 6000-6363P). Or complete ignition kit (p/n 6000-6701) Adjustment tabs may be sealed by Grundy Co Tech Officials. Boxes may be switched, changed, tested, or removed by Grundy Co. Tech Officials at any time. Car side harness must match all factory connections per diagram below with no modifications to allow tech officials to test system.

Connector: the 6 wire harness must be 24” long maximum and have a female 6 pin, weather pack

connector.

Six pin wiring diagram

a– ignition switch 12v (small red)

b – points pick-up (small white) brown gm boxes

c – coil negative (small black)

d – coil positive (small orange)

e – battery positive (large red)

f – battery negative (large black)

two pin optional for these two.

a – battery positive (large red)

b – battery negative (large black)

1. Only one ignition box allowed in car at any time. Car maybe wired for duel boxes but must have only one box in car while on track.

2. Box must be in clear view.　 Must be able to remove in five minutes,

Non - compliance with the specifications outlined herein may subject the participant to disqualification, loss of monies and points earned at the event. Furthermore, Failure to present car for inspection when requested to do so , or refusal to take steps requested by tech inspectors will be considered an admission of guilt and will be grounds for disqualification. Owner/driver must provide tools to remove parts. Any interpretation or deviation of these rules is left to the discretion of Grundy County Speedway tech officials, their decision is final.

Illustration A.1- Proper Driver side door plate installation

Illustration A.2- Proper Mounting Angles of Lap, Shoulder, and Sub-Straps

Lap Belt Angle     Sub Strap Angle      Shoulder Belt Angle

Illustration A.3- Proper Wrapping of shoulder Harness Belts

3-bar adjuster should be positioned as close possible to harness bar or snap-on/bolt-on bracket. This applies to both lap and shoulder belt points. The final wrap as pictured in figure 8 is mandatory. At least 4” of webbing material must extend out from the adjuster after this final wrap is completed.