This class of cars has been developed to permit those desiring to try stock car racing                                         and to do it at an absolute minimum cost.

ALL GRUNDY COUNTY SPEEDWAY GENERAL RULES APPLY.

# MODELS: 2006 and older V-6 powered front wheel drive American production passenger cars                           ( GM, Ford or Mopar) with a factory minimum wheelbase of 103 inches. NO CONVERTIBLES.

# BODIES:  Must be stock with no modifications, No gutting of doors, hood, or trunk allowed. Complete removal of all interior flammable materials. All glass except windshield must be removed this includes all lights, lenses and reflectors. Lexan windshield allowed. Sun roof openings must be covered with a steel sheet. Doors must be chained or welded shut. The stock dash and instrument cluster must remain in car. One stock mirror inside car, outside mirrors must be removed. Stock bumpers must be securely fastened to car.

# ENGINE: V-6 engines with a maximum displacement of 3.8 liters. The engine must be stock with no modifications. Engine must be original or a stock replacement for that make and model of car. NO TURBO OR SUPERCHARGERS. 12 volt electrical systems only. Battery must remain in stock location and must be securely fastened. OEM stock sensors, computers and computer chips only. No tuner performance chips allowed.

# COOLING SYSTEM: Any stock radiator allowed as long as it fits in the stock mounts. No racing radiators. Only the use of water for coolant, no antifreeze allowed. Over flow container under hood is mandatory.

# EXHAUST: Exhaust must exit behind the driver under the car. 3 inch maximum inside diameter for exhaust.

# FUEL TANK AND SYSTEM: The stock fuel tank may be used if it is located forward of the rear axle, stock fuel fill must remain. A fuel cell may be mounted in trunk and secured with a minimum of (2) 1/8” x1” steel straps and 3/8” bolts. Fuel pump must remain stock with no fuel lines running in the driver’s compartment. Stock fuel pressure regulators with no modifications allowed. No racing fuel or aftermarket fuel additives.

# DRIVELINE: Stock unmodified trans and transaxle as manufactured with all working forward and reverse gears. No high performance clutches or flywheels. No traction control devices. Automatic transmission only.

# SUSPENSION: The suspension must remain completely stock. The springs may not be cut or heated. OEM stock for your make and model springs, control arms, bushings, shocks, struts, spindles, etc only. Camber and caster must be stock. All steering system components must remain stock.

# BRAKES:  Stock brakes for your make and model only. Rotors, calipers, master cylinder must be stock. Anti lock brake system may remain functional. No brake bias adjusting devices.

# TIRES AND WHEELS:  Stock wheels only all 4 must be the same size, 14”, 15”, 16”. Tires must be steel belted DOT 60, 65, 70, 75 series radial tires only. You may use different tread width and series tires for staggering purpose. Over size lug nuts are mandatory. No racing type tire allowed.

Page 1

# ROLLCAGE: A 4 point roll cage made of  1-3/4” .095 wall minimum mild steel tubing is mandatory. If cage is mounted to floor pan a 10”x10”x1/8” steel plate must be used at cage mounting points. No portion of any bar of the cage can pass thru firewall to hook or stiffen the frame beyond the area of the driver’s compartment. No portion of the cage may be attached to or touch in any way to the rear strut towers or rear wheel wells. Only a maximum of two down bars may be used to support the main hoop.

# SEATS AND BELTS: Seat must be securely fastened to chassis and head area on back of seat must be braced to roll cage. Racing seat allowed. A 5 point racing harness in good condition (maximum 6 years old) is required.  And must be bolted to floor or cage with plates and minimum grade 5 bolts, nuts, and washers. Window net is required and must be up and latched any time car is on the track.

# SAFETY: A Snell SA 2010 or newer helmet, Fire retardant driving suit, eye protection, and gloves are mandatory. Head, neck restraints and fireproof shoes are highly recommended. Padding on center of steering wheel, window net, and working fire extinguisher mounted within the drivers reach is mandatory.

# Failure to present a car for inspection when requested to do so, or refusal to take steps requested by tech inspectors will be considered an admission of guilt and will be grounds for disqualification.

Any interpretations or deviation of these rules is left to the discretion of the tech officials their decision is final.

Page 2